## Environmental Legislative Requirements - New South Wales legislation

| Controlling legislation | Regulating authority | When does the legislation apply to Council? | Obligations imposed by the legislation | Application to this project |
| --- | --- | --- | --- | --- |
| Environmental Planning and Assessment |  |  |  |  |
| Environmental Planning and Assessment Act 1979 | Relevant local council Department of Planning and Environment Office of Environment and Heritage Environment Protection Authority Department of Primary Industries Planning Assessment Commission Independent Hearing and Assessment Panel Joint Regional Planning Panel | When Council is undertaking: - Any road and bridge construction and maintenance projects; - The erection (and sometimes demolition) of a building; - The alteration, demolition or addition to a building described in an environmental instrument as being a heritage item or within a heritage conservation area, a foreshore scenic protection area, a harbour foreshore preservation area or a beach front scenic protection area; - Any traffic generating developments (in relation to which a consent authority may be required to invite Council to comment); - Any road safety and traffic management works (such as the erection of traffic signs and barriers); - The change of use of a site; - The subdivision of a site; - Council consideration of approvals under section 138 of the Roads Act 1993; - Designated development (such as gravel and quarry pits); - Advertising; and/or - Storage or disposal of waste/recycled materials. There are exemptions for certain activities where Section 5.5 Duty to consider environmental impact and Section 5.7 Decision of determining authority in relation to certain activities of the Act do not apply. These include: - A modification of an activity, where the initially considered impact to the environment will not be substantially increased; - A routine activity such as maintenance of roads, bridge and tunnel cleaning, etc.; and/or - An activity that has been approved or is to be carried out by another determining authority after environmental assessment in accordance with the Act. | Part 5 Part 5 applies where development consent is not required for activities undertaken by or on behalf of a public authority (such as Council) under the provisions of the Act or other relevant legislation (such as for activities under ISEPP); however Council is still required to undertake an environmental assessment. Council is required to undertake self-assessment of its activities. There is a duty to consider environmental impact, which is undertaken through the preparation of a Review of Environmental Factors. OEH, EPA and the Department of Primary Industries concurrence may be required. Division 5.2 Council is required to obtain Ministerial approval for State significant infrastructure projects. State significant infrastructure (SSI) A project can be classified as SSI in two ways: - A SEPP may declare any particular development or class of development to be SSI. For example, under the State and Regional Development SEPP, development of port, wharf and boating facilities by or on behalf of a public authority with a value of $30 million or more is deemed to be SSI; and - Public authority activities that require an EIS. This includes development that would be assessed under Part 5 (i.e. does not require consent) but requires an EIS to be prepared. This includes all instances of designated development as listed in the EP&A Regulation. Critical State significant infrastructure Critical SSI can be declared by the Minister or in a SEPP. Currently particular Pacific Highway projects are listed as Critical SSI in the State and Regional Development SEPP. Environmental Impact Statement An EIS needs to be prepared when the proposed activity is classified as designated development and is considered likely to significantly affect the environment or threatened species, populations or ecological communities, or their habitats. Before preparing an EIS, Council must apply to the Secretary of the Department of Planning and Infrastructure (DPI) for environmental assessment requirements (Secretary’s Environmental Assessment Requirements - SEARs). The EIS must adequately satisfy the SEARs. Council must display the EIS for at least 30 days (extended to 45 days where the display overlaps with a school holiday period). Council must provide a submissions report in response to any submissions received during exhibition. Conditions The Minister may approve applications with modifications or conditions. Council is required to comply with any obligations in a statement of commitments. In relation to biobanking, the Minister may impose a condition that requires Council to acquire and retire biodiversity credits as specified in the approval. Part 4 Part 4 applies to development that requires consent and is not excluded from consent requirements by ISEPP or another environmental planning instrument. For example: maintenance depots, advertising signage, or the erection of administrative buildings (e.g. motor registry). State significant development (SSD) (Division 4.7) Ministerial approval is required. A project will be classified as SSD if the development: - Requires development consent under Part 4; and - Is specified in a SEPP or by the Minister. For example, the State and Regional Development SEPP lists activities such as mining, agriculture, manufacturing and warehouses with values in excess of a nominated figure (generally $30–50 million) as SSD. |  |
| SEPP 1 – Development Standards | Relevant local council Department of Planning and Environment | Any Council project for which development consent is sought and which does not comply with a “development standard”. | The relevant consent authority will consider whether the objection is well founded and whether the granting of consent would be consistent with the aims of SEPP 1. DPE concurrence may be required. |  |
| SEPP 19 – Bushland in Urban Areas | Relevant local council Department of Planning and Environment | Any activity that: - Involves the disturbance of bushland zoned or reserved for public open space purposes; or - Will be carried out on land in the metropolitan area that adjoins bushland zoned or reserved for public open space purposes. | Council must take into account the aims of SEPP 19 and/or the factors listed in clause 9 of SEPP 19. Council are exempt from the need to seek development consent under the SEPP. |  |
| SEPP 44 – Koala Habitat Protection | Relevant local council | Any development: - For which development consent is required; - That may affect “potential koala habitat” or “core koala habitat”; and - That is within one of the local government areas listed in Schedule 1 of the SEPP. | The relevant local council must take into account the guidelines made by DPE in relation to koala habitat before it can consent to a development application. Does not expressly apply to road projects assessed under Part 5. As a matter of practice, RMS generally considers impacts on “potential koala habitat” and “core koala habitat” during the course of the environmental assessment process for its projects. |  |
| SEPP 55 – Remediation of Land | Relevant local council Department of Planning and Environment | Where the zoning or rezoning of certain potentially contaminated land is being considered. Where consent is being sought for works on potentially contaminated land. Where remediation works are proposed | RMS may need to provide Council with a preliminary investigation. Certain categories of remediation work may be carried out without development consent, others need consent. Certain remediation works require a remediation plan to be approved. All remediation works are to be in accordance with applicable guidelines. Notice of remediation completion is to be given to council. |  |
| SEPP (Exempt and Complying Development Codes) 2008 | Relevant local council Department of Planning and Environment | Development that is exempt or complying development as identified in the SEPP - state wide application. | Provides assessment processes for development that complies with specified development standards by providing exempt and development codes that have state wide application. |  |
| SEPP 64 – Advertising and Signage | Relevant local council Department of Planning and Environment | When proposing to display an advertisement. When an applicant for development consent proposes to display an advertisement that: - Has a display area of greater than 20 square metres; and/or - Is higher than 8 metres above the ground and is within 250m of a classified road, and visible from that road. | General consent requirement for advertising. Despite the provisions of any other environmental planning instrument, the display of an advertisement on transport corridor land (which includes classified roads) is permissible with development consent in certain prescribed circumstances. Those circumstances include where the display of the advertisement in question is by or on behalf of RMS and is to be located on: - A road that is a freeway or tollway; - A bridge constructed by or on behalf of RMS on any road corridor; or - Land that is owned, occupied or managed by RMS and is within 250m of a classified road. The Minister for Planning and Environment is the consent authority for proposals in these categories. Proposals must be consistent with the DPE Transport Corridor Outdoor Advertising and Signage Guidelines. RMS concurrence is required for advertisements that are within 250 metres of a classified road and are greater than 20 square metres in display area or higher than 8 metres off the ground. |  |
| SEPP (State and Regional Development) 2011 | Minister for Planning Department of Planning and Environment | The aims of this Policy are as follows: - (a) to identify development that is State significant development, - (b) to identify development that is State significant infrastructure and critical State significant infrastructure, - (c) to identify development that is regionally significant development. | Dictates environmental assessment and approval processes for certain state and regionally significant projects. |  |
| SEPP (Infrastructure) 2007 | Department of Planning and Environment Minister for Planning | The Infrastructure SEPP 2007 (ISEPP) applies to many RMS developments. Consent and assessment requirements are amended for various types of development and locations. Roads and road projects where RMS development is generally permitted without consent include: - Eastern Distributor; - Cross City Tunnel; - Lane Cove Tunnel; - Tugun Bypass; - Liverpool—Parramatta Transitway; and - North-West Sydney Transitway Network. | No consent required The ISEPP removes most prohibitions and requirements for council consent where RMS is undertaking development for the purpose of a road or road infrastructure facilities. Such development undertaken by or on behalf of RMS is permitted without consent: There are limitations to the above exceptions where the works are in a National Park (or land reserved under the NPW Act), and does not require development consent or approval under State Environmental Planning Policy (Coastal Management) 2018, State Environmental Planning Policy (State Significant Precincts) 2005 or State Environmental Planning Policy (State and Regional Development) 2011. Further special provisions which may allow RMS to undertake development without consent or exempt development relate to: - Development within port facilities (cl 68 – 72); and - Public administration buildings (that is, offices or administrative buildings used by RMS) (cl 76 – 77). These provisions generally allow RMS to self asses its activities under Part 5 of the EP&A Act. Exempt development Certain types of minor development are identified as exempt development in Schedule 1. Exempt development may be carried out without consent under Part 4 and without assessment under Part 5 of the EP&A Act. To be exempt development must: - Be of minimal environmental impact; - Not be within critical habitat of an endangered species, population or ecological community; - Not be within a wilderness area; - Be in accordance with the Building Code of Australia and the Blue Book; and - Have no more than a minimal impact on any heritage item or conservation area. Consultation The ISEPP imposes consultation obligations where an RMS activity would impact on council activities, land or services or would be undertaken adjacent to particular types of land (Clauses 13-16). Consultation with Councils or relevant agencies is required if development would impact on stormwater services, council sewerage system, heritage items, or use substantial amounts of council water supply, disrupt pedestrian/vehicular traffic on Council land, change flooding patterns, involve works adjacent to a National Park, marine park, aquatic reserve or Sydney foreshore area or works in or over navigable waters. Council may be consulted regarding particular development types, such as: - Developments on land required for future classified roads, for which RMS Chief Executive Officer concurrence may be required (cl 100); - Sensitive development near busy road corridors; - Development adjacent to classified roads; or - Traffic generating developments. |  |
| Local Government Act 1993 | Department of Premier and Cabinet, Minister for Local Government Relevant local council | Generally RMS activities are carried out under Part 5 Division 5.1 or Division 5.2 of the EP&A Act, and therefore do not require local Council consent. This Act does not apply to those activities. RMS may need to lodge a development application with Council for particular activities or development (such as advertising signage). | Where RMS activities require a development application (DA) to be lodged with Council, RMS would lodge a Crown DA. If Council proposes to refuse approval or impose conditions it must notify RMS immediately, and Council must seek Ministerial approval. Council may not revoke or modify an approval without Ministerial consent. If a Council does not approve a Crown DA, it would go to the Minister for Planning for approval. Under the Act, Council has the power to give orders (such as an order to do or to stop doing something (e.g. ‘an order to prevent or repair environmental damage resulting from drainage or drainage works’). RMS is required to comply with any valid order. A failure to obtain or to comply with an approval and failure to comply with an order are offences. |  |
| Heritage |  |  |  |  |
| Heritage Act 1977 | Heritage Council of New South Wales Department of Planning and Environment Minister for Heritage Relevant local council | The Act applies: - If an activity would (or would be likely to) affect a place, building, work, relic, moveable object, precinct, or land listed on the State Heritage Register or that is in an Interim Heritage Order; - If an activity would involve disturbance or excavation on land that has or would be likely to have a relic; - To any building, work or relic listed, or within a precinct listed, on the State Heritage Register that is owned by RMS; and - To any item of environmental heritage (being a place, building, work, relic, moveable object, and precinct of State or local heritage significance) owned or occupied by RMS. | If an Interim Heritage Order or State Heritage Register listing applies, approval is required to: - Demolish the building or work; - Damage or despoil the place, precinct or land (or any part of the place, precinct or land); - Move, damage or destroy the relic or moveable object; - Excavate land to expose or move the relic; - Carry out development on land which is a heritage place, is within a precinct or on which a building, work or relic is situated; - Alter the building, work, relic or moveable object; - Display a notice or advertisement on the place, building, work, relic, moveable object, land or in the precinct; or - Damage, destroy or remove any tree or vegetation from the place, precinct or land. RMS may be required to obtain an excavation permit if a relic would (or would likely) be uncovered during excavation, or where excavation is planned on land where a relic has been discovered (see s 139). RMS must: - Notify the NSW Heritage Council and/or the Minister if a relic is discovered during works (see s 146 & 146A); - Maintain and/or repair items that it owns which are listed on the State Heritage Register to minimum standards set in the Heritage Regulation; and - Keep a Heritage and Conservation Register under s 170 which must be reviewed at least yearly |  |
| National Parks and Wildlife Act 1974 | Minister for the Environment Director General of the NSW National Parks and Wildlife Service National Parks and Wildlife Service | The Act is intended to provide for: - Conservation of nature; - Conservation of objects, places or features of cultural value; - Public appreciation, understanding and enjoyment of nature and cultural heritage; and - Management of land reserved under the Act (e.g. national parks, state conservation areas, nature reserves and historic sites among others). The Act applies to RMS if works would impact on: - Protected fauna or native plants or their environment; - Any Aboriginal object or place; - Any other item of cultural heritage on reserved land; - Threatened species, endangered populations or endangered ecological communities; - Critical habitat; or - An area subject to a conservation agreement. | Consent from the Minister for the Environment would be required for development in an area subject to a conservation agreement. RMS must obtain a licence or certificate in order to: - Harm any animal or pick any plant that is, or is part of, a threatened species, an endangered population or an endangered ecological community; - Damage critical habitat; - Damage any habitat of a threatened species, an endangered population or an endangered ecological community; - Harm, hold, kill or sell any protected fauna; or - Remove any water, or damage or remove any vegetation, rock, soil, sand or stone on reserved land. Authority may be required for works (pruning, felling, clearing and picking, etc.) on certain vegetation and for works on vegetation in protected areas. RMS must not harm any animal in a national park or heritage site. Aboriginal Heritage An Aboriginal Heritage Impact Permit is required to destroy, damage, move or deface any Aboriginal object or place. This is a strict liability offence, so penalties apply whether the harm was undertaken knowingly or unknowingly. RMS must notify the Director-General when an Aboriginal object is found. RMS staff should follow the Procedure for Aboriginal Cultural Heritage Consultation and Investigation (PACHCI) for all projects. Where a project is unlikely to affect Aboriginal cultural heritage this conclusion should be documented. |  |
| Contaminated Land |  |  |  |  |
| Contaminated Land Management Act 1997 | Environment Protection Authority Chief Executive of the Environment Protection Authority Minister for Energy and Environment | The Act applies to RMS if: - RMS owns land that is contaminated; - RMS contaminates land (whether or not RMS does or did own the land); or - EPA serves RMS (as a public authority) with a management order or an order to carry out activities on behalf of another person. - The object of the Act is to establish a process for investigating and remediating land that the EPA considers to be contaminated significantly enough to require regulation. Significance of contamination is assessed in relation to the approved use of the land. - Contamination of land under the Act means the presence of a substance (above the normal concentration in the locality) in, on or under the land that presents a risk of harm to human health or the environment. - Contamination can occur in a number of ways, including a change in the approved use of land (even if the contamination itself did not change); by the migration of contaminates into, onto or under the land from other land; or where pre-existing contamination was changed so that it became significant contamination. | RMS must comply with any EPA order to investigate or remediate land. RMS may be required to comply with an order to act on behalf of another person, as RMS is a public authority. If RMS incurs costs carrying out such an order, it may recoup these costs. RMS has a duty to report contamination to the EPA. This applies where RMS caused contamination or where RMS owns land that is contaminated. This duty includes instances where RMS ought reasonably to have been aware of the contamination. RMS is generally required to assess remediation work in the broader EIA for a project (e.g. under Part 5 for the activity being undertaken). However, if contaminated land remediation was required to be completed separately from a road project, RMS would consider SEPP 55 – Remediation of Land and may require consent depending on the type of remediation work. |  |
| Waste Management and Licencing |  |  |  |  |
| Waste Avoidance and Resource Recovery Act 2001 | Office of Environment and Heritage Minister for the Environment and Energy | The legislation may apply where RMS works with OEH to contribute to the development of waste strategies. Under the Act the Director-General can develop extended producer responsibility schemes. If developed these could apply to RMS. | No obligations are currently imposed by the Act. However, the NSW Waste Reduction and Purchasing Policy (WRAPP) applies to RMS and requires the RMS Annual Report to include a statement about progress made towards implementing the WRAPP requirements. RMS must also provide a biennial WRAPP report to OEH. The WRAPP imposes requirements in relation to waste avoidance, recycling and purchasing of materials with recycled content. |  |
| Biodiversity Conservation |  |  |  |  |
| National Parks and Wildlife Act 1974 |  | See commentary above under Heritage. |  |  |
| Fisheries Management Act 1994 | Department Primary Industries Minister for Regional Infrastructure and Services Minister for Primary Industries | The Act applies to Council if activities would: - Occur in an area affected by a habitat protection plan or in an aquatic reserve; - Involve dredging or reclamation, or the construction, alteration or modification of a dam, weir or reservoir; - Be likely to harm threatened species, populations, ecological communities or their habitat or critical habitat; - Be likely to disturb a gravel bed where trout or salmon would be likely to spawn; or - Be likely to harm mangroves, seagrasses or declared marine vegetation. Dredging means excavating water land and removal of woody debris, snags, gravel beds, cobbles, rocks, boulders, rock bars or aquatic vegetation from water land. Reclamation means draining water from, filling or depositing material on water land for the purpose of reclamation. Water land means land submerged by water, permanently or intermittently, whether forming a natural or artificial body of water. | Under the Act the Minister may prepare habitat protection plans or declare aquatic reserves. Council must: - Have regard for habitat protection plans and is to notify the Minister for Regional Infrastructure and Services and the Minister for Primary Industries if an activity is inconsistent with a habitat protection plan; - Not determine or carry out an activity under Part 4 or 5 of the EP&A Act in an aquatic reserve without considering the objects of the Act, any management plan objectives and the permissible use of the area under the Act. Before granting consent RMS must obtain the concurrence of the Minister; and - Obtain the concurrence of the Minister before granting approval or carrying out work if an EIS under Division 3 of the EP&A Act is required. Council must not: - Carry out dredging or reclamation without giving written notice to the Minister and considering the Minister’s comments; - Harm mangroves, seagrasses or declared marine vegetation (identified in the Regulations) without a licence or permit from the Minister; - Disturb a gravel bed where trout or salmon are likely to spawn without a licence or permit; - Construct, alter or modify a dam, weir or reservoir without notifying the Minister. The Minister may require RMS to conduct works to enable fish to pass through or over the dam, weir or reservoir (e.g. construct a fishway or fish bypass); - Block a waterway with a net, weir, floodgate or other obstruction so that fish could stranded and their passage obstructed without a permit; or - Harm critical habitat or interfere with or harm threatened species, populations, ecological communities or their habitat without a permit or licence. Council may be required to implement recovery and threat abatement plans, and consider the plans when making decisions. Council must consider any critical habitat land which it holds when exercising its functions in relation to the land. Council may enter into a joint management agreement with the Minister for the management, control, regulation or restriction of an action that is jeopardising the survival of a threatened species, population or ecological community. |  |
| Biodiversity Conservation Act 2016 | Office of Environment and Heritage | The Biodiversity Conservation Act 2016 (BC Act) repealed the Threatened Species Conservation Act 1995, the Native Vegetation Act 2003, the Nature Conservation Trust Act 2001 and the animal and plant provisions of the NP&W Act. The purpose of the BC Act is to maintain a healthy, productive and resilient environment for the greatest well-being of the community, now and into the future. Introduced in 2016, the BC Act aims to deliver ecologically sustainable development in New South Wales and address the loss of biodiversity via a biodiversity offsets scheme and private land conservation agreements. | Local government is a key partner in the conservation and management of biodiversity and threatened species in NSW. The BC Act, together with the Biodiversity Conservation Regulation 2017, outlines the framework for assessment and approval of biodiversity impacts associated with developments that require consent under the Environmental Planning and Assessment Act 1979. Any new application for development consent or modification to an approved development under Part 4 of the Environmental Planning & Assessment Act 1979 (EP&A) (not including State Significant Development) will be subject to the biodiversity assessment requirements of the Biodiversity Conservation Act 2016. The threatened species ‘test of significance’ is used to determine if a development or activity is likely to significantly affect threatened species or ecological communities, or their habitats. If a proposed development under Part 4 of the EP&A Act is likely to significantly affect threatened species, a biodiversity development assessment report must be prepared by an accredited assessor. The biodiversity offsets scheme will apply. If a proposed activity under Part 5 of the EP&A Act is likely to significantly affect threatened species, and the proponent does not opt in to the Biodiversity Offsets Scheme, a Species Impact Statement (SIS) must be prepared. |  |
| Biosecurity Act 2015 | Department of Primary Industries | On 1 July 2017 the Biosecurity Act 2015 replaced the Noxious Weeds Act 1993. The new Biosecurity Act 2015 combines 14 different pieces of legislation into a single Act of law. Under the Noxious Weeds Act all landowners, including Council, have a responsibility to control noxious weeds on their property. Under the Biosecurity Act the same responsibility will apply and will be known as a General Biosecurity Duty. | Council’s Urban Weeds Program and the process for inspecting private properties for invasive weeds continues unchanged. Council also maintains its current approach to education and enforcement relating to invasive weeds. Council maintains the current process for issuing Weed Control Notices. The main differences will be the terminology used and that Orders will be issued under the Biosecurity Act. They will be known as Biosecurity Directions. |  |
| Water Act 1912 [This Act is being progressively repealed by the Water Management Act 2000 and will eventually be phased out entirely] | Department of Primary Industries, Office of Water | Taking and using water from a river or a lake. Taking and using water from a dam, lock, reservoir, weir, regulator, flume, race, channel, cutting, well, excavation, tunnel, pipe, sewer, machinery or appliance (“a work”). Constructing any work located on the bank of a river or lake, or within a floodplain, or affecting the quantity of water flowing in a river or a lake (“a controlled work”). Licensing requirements for the sinking of bores do not apply where the bore is to be sunk, enlarged, depended or altered by the Crown – see section 112(1)(b). | A licence or permit is required for construction or use of a “work” for certain purposes. The application for a licence is to include certain information on how and what quantity of water is to be extracted. An approval is required for controlled works and may be granted with conditions attached. An exemption in relation to roads arguably still applies – see clause 4 of the Water (Part 8-General) Regulation 1995. It is an offence to fail to comply with the licensing and approval provisions, and certain directions that may be issued by the Ministerial Corporation under those provisions. There are prohibitions on polluting rivers and lakes and interfering with sub-surface water. |  |
| Water Management Act 2000 [This Act is progressively replacing the Water Act 1912] | NSW Department of Primary Industries Office of Water Minister for Finance and Services Minister for Primary Industries | The Act applies where: - A water management plan applies to RMS; and/or - RMS proposes to interfere with an aquifer or undertake works which affect waterways (taking water, draining land or affecting flooding). | A water management plan may apply to a particular public authority, or a group of or all public authorities. RMS is required to consider any water management plan which applies to it when exercising its functions. RMS may require a licence to take water from water sources (such as rivers, lakes, estuaries, groundwater and coastal waters). However, exemptions to the need for a licence or approval are set out in Schedule 5 of the Water Management Regulation 2011. RMS will generally be exempt from requiring a licence (or an approval) where it takes water required for road construction and road maintenance. RMS may require an approval to undertake the following: - Construct and use a water supply work (to take, store, divert, convey or impound water such as pumps, dams, pipes, channels, weirs or levees); - Construct and use a drainage work (for the purpose of draining water from land); - Construct and use a flood work (in the vicinity of a waterway which is likely to have an effect on water flows and / or flooding); or - Undertake an aquifer interference activity (such as penetrating, interfering with, obstructing or taking water, sand or road base from an aquifer. Within the Hunter Valley RMS must not construct protective works (to protect works against flooding) without the Minister’s approval and must comply with any notice from the Minister. The Act also sets out the powers of water supply authorities, for instance, to interfere with public roads and sewers, drains or tunnels under public roads. |  |
| Wilderness Act 1987 | Office of Environment and Heritage Minister for Energy and Environment National Parks and Wildlife Service | Any road or bridge construction and maintenance projects to be carried out on land within a “wilderness area” (as declared under this Act or the NPW Act). | Approval required to carry out development in a wilderness area that is subject to a wilderness protection agreement or a conservation agreement. |  |
| Pollution |  |  |  |  |
| Protection of the Environment Operations Act 1997 | Environment Protection Authority (EPA) | The Act applies if Council proposes an activity that would: - Cause or have the potential to cause water, air, noise or land pollution; - Be a ‘scheduled activity’ and so, requires an environmental protection licence (see Part 3 and Schedule 1 of the Act); - Involve the creation, disposal or transport of waste; or - Be likely to harm the environment (whether wilfully or negligently). - The Act also applies if a pollution incident is caused by RMS’ activities or occurs on its land. | Environmental Protection Licences: Council must obtain an environmental protection licence before undertaking any scheduled activities. The scheduled activities set out in Schedule 1 that are most relevant to RMS include: - Concrete works (cl 13); - Crushing, grinding or separating materials (cl 16); - Land-based or water-based extractive activities, such as extraction, dredging, quarrying, processing or storage (cl 19); - Dealing with certain types of waste (see below); and - Road construction, widening or re-routing (but not maintenance or operation) where this results in 4 or more traffic lanes (not including bike lanes or lanes used for entry and exit). To activate this clause, the road must be at least 1km to 5km in length depending on whether it is in a metropolitan or non-metropolitan area and on a freeway, tollway or main road (cl 35). An environmental protection licence may require Council to undertake an environmental audit under the terms of the licence. Council must comply with any monitoring requirements which are a condition of an environmental protection licence. Waste Waste is classified into various categories, and an environmental protection licence may be required for some of these. Depending upon classification and/or other factors, such as the volume of waste generated, and whether waste is to be transported off-site, then an environmental protection licence may be required for those activities. The “RTA stockpile exemption 2011” exempts Council from the need to hold a licence for storing RMS excavated road materials in stockpiles managed in accordance with the Stockpile Site Management Guideline. This exemption applies until it is revoked by the EPA. Certain types of higher risk wastes must be tracked when they are transported (categories include but are not limited to explosive, corrosive, toxic, poisonous and flammable waste). All waste must be stored on premises in an environmentally safe manner. Waste which is being transported must comply with a number of requirements including avoiding spillage, covering waste, safely securing containers, no mixing or transporting together of incompatible wastes, and waste must be capable of being sampled by the release of valves on the container. For some wastes only licensed transporters can be used. In some cases levies may be required to be paid and/or the activities may be regulated (for example, waste may need to be stored in certain ways, and certain records kept) even though the activities may not require a licence. If Council consigns waste for transport it must: - Hold a consignment authorisation in relation to the waste (and retain these records for 4 years); - Obtain a waste transport certificate (and retain these records for 4 years); - Ensure the transporter is appropriately licensed (where necessary); and - Ensure the waste facility is legally allowed to accept the type of waste concerned. Pollution Council must not cause water, air, noise or land pollution. If pollution is caused the EPA may issue a clean-up notice, prevention notice, prohibition notice or noise control notice. Council must comply with any notice received from the EPA. Where Council becomes aware of a pollution incident which might cause material harm to the environment (that is, harm to human health or safety, harm to ecosystems or property damage of over $10,000) Council has a duty to report the incident to the following agencies as soon as is practicable: - EPA (and other relevant “regulatory authority” which may be a public authority); - RMS; - Ministry of Health; - WorkCover Authority; and - Fire and Rescue NSW. The duty to report applies to: - A person carrying on the activity which causes a pollution incident; - The employer of that person (who becomes aware of the incident); and - The occupier of premises where an incident occurs. It is good practice for Council, as a government agency, to report an incident on an RMS road (unless we know it has already been reported). Council must maintain its vehicles so as to prevent excessive air impurities being emitted, and not sell any vehicle that emits excessive air impurities (ss155 – 162). Pollution incident response management plan Where it holds an environmental protection licence (or where the EPA directs) Council will need to develop a pollution incident response management plan (PIRM plan) which include: A detailed description of the action to be taken to reduce or control any pollution if a pollution incident occurs. Contact details and communication procedures for co-ordinating with relevant authorities regarding action taken to combat the pollution caused by the incident. The PIRM plan must be prepared, kept and tested in accordance with the regulations, and implemented immediately if an incident occurs. Publication of monitoring results Council may be required to undertake monitoring under an environmental protection licence. The results of monitoring must be displayed publicly and prominently on the RMS website within 14 days of obtaining the data. |  |
| Environmentally Hazardous Chemicals Act 1985 | Office of Environment and Heritage Environment Protection Authority Minister for Energy and Environment | When Council undertakes a prescribed activity, such as: manufacturing, processing, keeping, distributing, conveying, using, selling or disposing of chemicals or chemical waste. | If Council proposes to undertake a prescribed activity, an application will need to be made to the EPA for a licence. When maintaining, demolishing or dealing with soil wastes from work on timber bridges, Council should consider the Scheduled Chemical Waste Chemical Control Order under the Act. |  |
| Work Health and Safety Act 2011 | WorkCover Minister for Finance and Services | Storage and handling of dangerous goods. (The WHS Act has broader implications for RMS which are discussed below). | Schedule 1 confirms the application of the Act to the storage and handling of dangerous goods, even when this does not occur on a worksite. |  |
| Dangerous Goods (Road and Rail Transport) Act 2008 | WorkCover Office of Environment and Heritage Environment Protection Authority Minister for Finance and Services Minister for Energy and Environment | When Council is transporting dangerous goods by road or rail. A list of common dangerous goods is available in the Australian Code for the Transport of Dangerous Goods by Road or Rail (7th edition) (known as the ADG Code) including items such as ammunition, bombs and other weapons, fireworks and flares and flammable/explosive chemicals including petrol amongst many others. This document also lists designated “goods too dangerous to be transported”. The regulation also identifies “designated dangerous goods” to include Class 1 explosives, infectious substances and radioactive material. | Council must: - Only transport dangerous goods by road or rail in a safe way; - Obtain a licence for vehicles to be used to transport dangerous goods; - Only consign dangerous goods for transport in a licenced vehicle; - Ensure that a person driving a vehicle transporting dangerous goods is licensed under the regulations; and - Comply with a notice issued by an authorised officer (including EPA, WorkCover and police). Council must not consign dangerous goods for transport: - If appropriate safety equipment and transport documentation is not carried; - If they are not marked, packaged, stowed, and segregated in accordance with the regulations and the ADG Code; and - That are identified as “too dangerous to be transported”. Where Council operates a weighing facility, records made of the mass of a vehicle transporting dangerous goods may be admissible in any proceedings under the Act. Council may recover costs incurred as a result of an incident relating to the transport of dangerous goods, such as an escape of dangerous goods or an explosion or fire involving those goods. |  |
| Pesticides Act 1999 | Minister for Energy and Environment Environment Protection Authority | The Act applies when Council is in possession of, supplies or uses pesticides. “Using” pesticides includes: applying, spraying, spreading, dispersing, storing or preparing a pesticide for use. | Council employees must not use pesticides in a way that: - Injures or would be likely to injure a person; - Damages or would be likely to damage the property of another person; - Harms a non-target plant or animal (non-target means a plant or animal on which pesticide use has not been approved on the label or permit); - Harms any animal or plant (if there is no approved label or permit for the pesticide); or - Materially harms any threatened species (listed under the Threatened Species Conservation Act 1995) or any protected fauna (listed under the National Parks and Wildlife Act 1974). Under Division 3 of the Act: - Council may be required to obtain permits for the possession/use of unregistered pesticides; - People using pesticides are required to read approved labels/permits and follow all instructions on approved label unless authorised by a permit (though use of a lower concentration than listed on an approved label is allowed); - Pesticides must be stored with appropriate label; - Restricted pesticides may only be used if authorised by a certificate of competency or a pesticide control order; and - Council employees may be required to apply to the EPA for a certificate of competency to use or possess restricted pesticides. Under the Pesticides Regulation 2009 Council must: Prepare and make available a Pesticide Use Notification Plan to notify the community about pesticide applications in public places; and Maintain a record of the use of pesticides. Many pesticides because of their flammability, combustibility or toxicity, are classified as dangerous goods. In this instance the Road and Rail Transport (Dangerous Goods) Act 1997 will also apply. |  |

## Commonwealth legislation

| Environmental Planning and Conservation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Controlling legislation | Regulating authority | When does the legislation apply to RMS? | Obligations imposed by the legislation | Application to this project |
| Environment Protection and Biodiversity Conservation Act 1999 | Department of Environment and Energy | Any road or bridge construction and maintenance projects where the project will have, or is likely to have, a significant impact on a matter of National Environmental Significance (NES). NES include: - World Heritage properties; - National heritage places; - Ramsar wetlands; - Commonwealth listed threatened species and communities; - Commonwealth listed migratory species; - Nuclear actions; and Commonwealth marine areas. | All environmental assessment undertaken for projects under Part 4 and 5 of the EP&A Act must consider potential impacts in relation to NES matters as well as any Commonwealth land. In cases where a significant impact is considered likely, a referral should be made to the Department of Environment and Energy. The approval of the Commonwealth Environment Minister may be required in addition to any NSW approvals. Processes for assessment of controlled activities under the EPBC Act are affected by the following: - The Bilateral Agreement (between the Commonwealth and NSW relating to environmental impact assessment); - Environment Protection Biodiversity Conservation Act, 1999: Guide to Implementation in NSW (Department of Planning, 2007); and - Part 13 permits – various activities in relation to listed species and communities. |  |
| Aboriginal and Torres Strait Islander Heritage Protection Act 1984 | Department of Environment and Energy | Any Council site subject to a declaration under the Act in relation to an area or object of Aboriginal significance. Any Council activity with the potential to affect an area or object of Aboriginal significance or any Aboriginal remains. | s20 – Report any discovery of Aboriginal remains to the Federal Minister for Sustainability, Environment, Water, Population and Communities. s22 – Comply with the provisions of any declaration in relation to a significant Aboriginal area or object. | Requirements for the safeguarding or management of any aboriginal area, object or remains are addressed in S.2.9 of the CEMP. |