| Task | Hazard | Risk (Pre) | Controls | Risk (Post) | Responsibility | CCVS Code |
| --- | --- | --- | --- | --- | --- | --- |
| Crack Stitching with Thor Helical Bars [Cut slots into masonry, install Thor helical bars, and grout to stitch cracks in brick or block walls.] | Cut slots into masonry, install Thor helical bars, and grout to stitch cracks in brick or block walls. | High | Use a wet-cut grinder or SDS drill to cut slots at correct depth and spacing. Check for hidden services before cutting — use a cable and pipe detector. Set up full dust controls: water suppression, vacuum shroud, or both. Wear a P2 respirator when cutting or mixing grout in enclosed or low-air spaces. Barricade the work zone to keep others clear of flying debris and dust. Mix grout to maker's ratio — do not add extra water. Inject grout from the back of the slot forward to avoid air pockets. Insert helical bars while grout is wet — push to full depth. Check bar spacing matches the engineer's or maker's layout drawing. Allow grout to cure to maker's time before pointing or loading the wall. Store helical bars flat and dry to prevent kinks or rust before use. Dispose of grout waste and slurry as per site and local rules. Inspect tools before use — check discs, chucks, and leads for damage. Keep bystanders at least 3 m back from any cutting work. Report any new or widening cracks found during work to the supervisor at once. Admin: Confirm engineer or maker's specification is on site before starting. Complete a pre-start check of all tools and PPE each day. Record bar locations, grout batch numbers, and cure times in the site log. Brief all workers on the task hazards and controls before starting. Review MSDS for grout product — keep a copy on site. PPE: P2 respirator Safety glasses or goggles Face shield when cutting Hearing protection Cut-resistant gloves Steel-capped boots Hard hat Hi-vis vest HOLD POINT — do not start until: Confirm crack width, depth, and cause before cutting slots — get approval to proceed. Inspect grouted slots before pointing — ensure full bar embedment and no voids. Final check: confirm all bars are set and pointing is flush before sign-off. STOP WORK if: Crack is active or still moving — stop and get structural advice. Hidden services are struck or suspected — stop all cutting at once. Grout will not bond or slots are wet from rising damp — stop work. Wall shows signs of instability or bulging — evacuate and report. Dust controls fail and no respiratory protection is available — stop cutting. | Low | SUP — Confirm spec is on site, approve hold points, check grout and bar records daily. WKR — Follow cutting, grouting, and bar-install steps, wear PPE, and report issues at once. | N/A |