| Task | Hazard | Risk (Pre) | Controls | Risk (Post) | Responsibility | CCVS Code |
| --- | --- | --- | --- | --- | --- | --- |
| Lead Paint — Surface Preparation or Removal [Sanding, scraping, or removing paint from surfaces where lead is present or assumed.] | Sanding, scraping, or removing paint from surfaces where lead is present or assumed. | High-6 | Wet methods only no dry sanding or scraping Containment sheeting to prevent spread Test for lead before work or assume present and apply lead-safe controls Decontamination procedures remove coveralls before leaving work area, wash hands and face before eating or drinking Waste disposal per lead paint waste guidelines P3 for extensive work or elevated airborne concentrations PPE: P2 minimum, disposable coveralls, cut-resistant gloves, eye protection STOP WORK if: Remove from exposure and rectify if dust controls fail | Low-2 | SUP — Wet methods only, HEPA vac, confirm or assume lead, decontamination before leaving area, P2 minimum. | N/A |