Walker St Roof HVAC Maintenance

| PCBU: | [Insert PCBU here] | Workplace location: |  |
| --- | --- | --- | --- |
| Works Manager: |  | Date SWMS provided to PC: | 31/03/2026 |
| Work activity: | Walker St Roof HVAC Maintenance | Principal Contractor (PC): | [Insert PCBU here] |
| High Risk Construction Work (HRCW): ☑ Risk of a person falling more than 2 metres ☐ Likely to involve disturbing asbestos ☐ Work in or near a shaft or trench deeper than 1.5 m or a tunnel ☐ Work on or near chemical, fuel or refrigerant lines ☐ Tilt-up or precast concrete elements ☐ Work on a telecommunication tower ☐ Temporary load-bearing support for structural alterations or repairs ☐ Use of explosives ☐ Work on or near energised electrical installations or services ☐ Work on, in or adjacent to a road, railway, shipping lane or other traffic corridor in use by traffic other than pedestrians ☐ Demolition of load-bearing structure ☐ Work in or near a confined space ☐ Work on or near pressurised gas mains or piping ☐ Work in an area that may have a contaminated or flammable atmosphere ☑ Work in an area with movement of powered mobile plant ☐ Work in areas with artificial extremes of temperature ☐ Work in or near water or other liquid that involves a risk of drowning ☐ Diving work |  |  |  |
| Person responsible for ensuring compliance with SWMS: | [Insert Supervisor name here] | Date SWMS received: | 31/03/2026 |
| What measures are in place to ensure compliance with the SWMS? | Toolbox meetings, SWMS sign off, job observations and supervision review. If issues with the SWMS or new hazards are identified, the supervisor must be notified. When changes are made to SWMS, it will be communicated to all workers. |  |  |
| Person responsible for reviewing SWMS control measures: |  | Date SWMS received by reviewer: | 31/03/2026 |
| How will the SWMS control measures be reviewed? | The control measures implemented will be reviewed and if necessary, revised annually or if work methods change, the control measures are not effective in controlling the risk, a new hazard/risk is identified or following an incident. The SWMS will be reviewed in consultation with workers and/or others who may be affected by the SWMS. Any changes to the SWMS will be communicated with workers at induction, daily pre-starts and toolbox talks. |  |  |
| Reviewer’s signature: |  | Review date: | 31/03/2026 |
| This SWMS must be kept and be available for inspection until the high-risk construction work to which this SWMS relates is completed. If the SWMS is revised, all versions should be kept. If a notifiable incident occurs in relation to the high-risk construction work in this SWMS, the SWMS must be kept for at least 2 years from the date of the notifiable incident. |  |  |  |

PRE-REQUISITES TABLE

| Must be in place before starting work |  |  |  |
| --- | --- | --- | --- |
| PPE — All Persons on Site | • steel-capped footwear • hi-viz shirt or vest • eye protection • hearing protection (>85dB) • cut-resistant gloves • Hard hat where overhead risk present | Additional PPE | • Fall prevention hierarchy applied: eliminate > isolate > minimise • Guardrails preferred. Fall restraint before fall arrest • Rescue plan documented for all harness work • Working at Heights licence/training verified before elevated work |
| Licences / Qualifications | • Fire warden designation — person assigned before work starts • Fire extinguisher — 9L water or 9kg CO2 within 3m of work • Competent person — pre-use inspection of fall arrest equipment • Rescue plan — documented before work above 2m begins • Rescue equipment — on site and ready for immediate deployment before any elevated work starts • Ladder secured — tied or footed at base before use | Permits & Approvals | • Hot Work Permit — mandatory before welding or cutting in any building • Fire watch — 30 minutes minimum after hot work ceases • Working at heights permit — signed before each elevated task • Engineer confirmation — roof structure adequate at transfer point where EWP-to-roof transfer is planned • Roof access permit — before any worker accesses roof • Crane/hoist permit — where lifting plant to roof or mezzanine |
| Plant & Equipment | • Elevated Work Platform (EWP) • Scissor lift | Hazardous Substances | • Hydraulic fluid — SDS on site — skin sensitiser |
| Consultation | • Workers consulted during SWMS development • Site-specific induction completed before work commences • Toolbox talk conducted at start of each shift | Legislative Basis | • WHS Act 2011 (NSW) • WHS Regulation 2017 (NSW) • SafeWork NSW Codes of Practice • SafeWork NSW — Hot Work Code of Practice |

Brief description: Walker St Roof HVAC Maintenance

STEP-BY-STEP SAFE WORK METHOD TABLE

| Step | Task | Hazard | Risk (Pre) | Controls | Risk (Post) | Responsibility | CCVS Code |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1 | Establish exclusion zone and position scissor lift at ground level | • Crush injury — scissor lift hydraulic failure or uncontrolled descent • Struck by moving vehicle — car park access not fully cleared • Entrapment — worker contact with scissor lift mechanism during positioning • Loss of stability — uneven or contaminated car park floor bearing scissor lift load | High (6) | Engineering Controls: • Check car park floor for oil, water, debris, or soft spots within scissor lift footprint and 1 m radius before positioning. • Position scissor lift only on level, solid, dry concrete. Do not place on gravel, asphalt, or suspect areas. • Establish exclusion zone: erect barriers (Class A fencing or tape) minimum 5 m perimeter below roof edge transfer point. Display warning signs. • Confirm car park access closed to all vehicles. Place 'Do Not Enter' signs at entry points. Verify no vehicle movement before lift operation. • Check scissor lift base for hydraulic leaks, visible damage, or loose fasteners before positioning. Do not use if defective. • Operate scissor lift controls only from ground controls while positioning. Confirm platform is fully lowered and locked before workers approach. • Brief all workers on exclusion zone boundaries and scissor lift hazards. No entry within exclusion zone during lift positioning or raising. • Position lift personnel only — maximum two workers spotting. All others remain outside exclusion zone. Administrative Controls: • Supervisor to confirm vehicle access closure and briefing completion before lift positioning starts. • Record floor bearing condition check (signed by supervisor) before lift platform is raised. | Medium (3) | • SUP — Verify exclusion zone complete, barriers intact, access controlled before work starts • WKR — Set up and maintain barriers, stop work if zone breached, report access issues | ⚠️ HOLD POINT 1.1 1. Do not raise scissor lift until: ground floor base bearing is confirmed stable (checked and signed by supervisor), exclusion zone is fully established and car park access is clear. 🛑 STOP-WORK TRIGGER • Hydraulic leak, torn seals, or visible damage detected on scissor lift base or cylinders. • Car park floor shows settlement, collapse, or soft spots within scissor lift footprint. • Vehicle access cannot be confirmed closed or vehicle enters exclusion zone. |
| 1.2 | Replace HVAC filters on four rooftop AHUs | • Fall from height — worker proximity to unprotected roof edge during AHU access • Crush injury — rotating fan blade contact during filter removal • Laceration or abrasion — sharp filter frame edges or metal louvers • Inhalation of dust or contamination — aged filter material disturbance | High (6) | Engineering Controls: • Do not access roof or approach AHU until EWP is positioned, stabilised, and anchor point confirmed by supervisor. • Before filter removal, switch HVAC system to OFF and lock isolation switch in place — verify no fan rotation for 30 seconds. • Wear disposable chemical-resistant gloves and dust mask (P2) when handling aged filter material. • Remove spent filter slowly into closed bag — do not shake or release dust into air. • Check filter housing interior for loose debris, corrosion, or damage before installing new filter — photograph defects if visible. • Install replacement filter with gasket seal fully seated in frame — confirm frame clip engagement on all corners. • Attach safety harness to EWP anchor point before accessing any AHU edge — maintain harness tension during the entire task. • Position EWP to maintain 1.2 m minimum clearance from unprotected roof perimeter edge — do not overreach beyond platform edge. Administrative Controls: • Obtain current method statement or permit-to-work from facilities or site supervisor before starting work. • Verify EWP operator holds current licence and has completed safety briefing on roof hazards. • Record filter replacement completion — photograph each AHU post-installation to confirm filter type and seal fit. • Confirm HVAC system is restored to operational status — no restart until all workers have left roof. | Medium (3) | • SUP — Supervise Replace HVAC filters on four • WKR — Perform Replace HVAC filters on four per SWMS | ⚠️ HOLD POINT 1.2 1. Do not start filter replacement until: EWP is positioned, anchor point is confirmed by supervisor, and HVAC system is isolated with lock-off in place 2. Do not leave roof until: all four filters are installed, sealed, and frames are checked for correct fit; AHU housings are visually clear of loose debris 🛑 STOP-WORK TRIGGER • EWP anchor point fails, becomes loose, or cannot be confirmed before work starts • HVAC system cannot be isolated or fan rotation is still present after isolation • Worker loses balance, harness becomes detached, or edge protection is breached RISK CODE:WAH-H6 |
| 1.3 | Verify scissor lift access arrangement and roof entry conditions | • Fall from height — unguarded or degraded roof edge protection • Fall from height — platform misalignment at transfer point • Entrapment or crush injury — guardrailed gate malfunction or obstruction • Struck by falling object — unsecured tools or materials on roof during inspection | High (9) | Engineering Controls: • Check roof edge protection is continuous, secure, and undamaged along entire transfer zone before lift platform positioning starts. • Verify scissor lift platform aligns flush with roof threshold — no gap or step — and test gate opening and closing freely without obstruction. • Confirm guardrail height 1.0–1.2 m and load capacity 1.2 kN/m at transfer point; do not proceed if integrity is compromised or non-compliant. • Barricade and exclude all personnel below work zone using signage, tape, and verbal brief until roof entry task is complete. • Brief occupant exclusion authority and confirm building occupants are not in exclusion zone before lift deployment. • Check all loose tools and materials are secured or removed from scissor lift platform before raising to transfer height. • Lower scissor lift platform and confirm gate closes fully after final occupant exits roof; do not leave platform elevated unattended. Administrative Controls: • Site supervisor to conduct pre-use briefing with all occupants on transfer point hazards and emergency procedures. • Document roof edge condition and platform alignment check in site diary before lift deployment. • tell building manager of exclusion zone extent and duration; maintain records of occupant briefing. | Medium (6) | • SUP — Supervise Verify scissor lift access arrangement • WKR — Perform Verify scissor lift access arrangement per SWMS | ⚠️ HOLD POINT 1.3 1. Do not raise scissor lift platform until: roof edge protection is checked and documented as compliant, and platform-to-roof alignment is verified by supervisor. 2. Do not allow occupant transfer until: gate mechanism is tested open and closed, exclusion zone is confirmed clear, and occupant briefed on transfer point layout. 🛑 STOP-WORK TRIGGER • Roof edge protection shows cracks, gaps, or separation from structure. • Scissor lift platform gap exceeds 50 mm or guard height falls below 1.0 m. • Gate sticks, binds, or cannot close fully, or exclusion zone is breached by personnel. RISK CODE:WAH-H6 |
| 1.4 | Raise scissor lift and align platform at roof transfer point | • Fall from height — platform misalignment or instability at elevation • Crush injury — scissor lift hydraulic failure during ascent • Shear or entrapment — platform contact with roof edge or obstruction during raising • Struck by falling object — tools or materials dislodged during platform movement | High (9) | Engineering Controls: • BASE CONTROLS TAGGED OUT before any worker initiates transfer step — non-negotiable. Ground spotter confirms tag-out complete and calls 'TRANSFER CLEAR' before worker moves • Confirm horizontal gap between EWP platform and roof landing does not exceed 300mm and vertical step does not exceed 100mm — re-measure if EWP has repositioned • Worker attaches lanyard to certified structural roof anchor before disconnecting from EWP anchor — worker must not be unattached at any point during transfer • Ground spotter maintains visual contact with transferring worker throughout and confirms safe landing before clearing next worker • Confirm scissor lift base is level and on firm, stable ground before raising platform. • Check all scissor lift hydraulic hoses and connections for leaks or damage before starting. • Raise platform slowly to roof transfer height — stop and check alignment at 50% height. • Position platform squarely with guardrailed gate opening — use laser level or theodolite to confirm alignment. • Test platform stability at full height — rock platform gently side-to-side and confirm no movement or sway. • Operate transfer gate fully open and fully closed — confirm smooth operation and no binding. • Prohibit workers from stepping on or near platform edge until alignment and stability confirmed. • Station spotter on ground — confirm line of sight to platform always during raising. Administrative Controls: • Verify operator holds current scissor lift competency card (RII or equivalent) before starting. • Confirm roof structure engineer assessment is available — state load capacity at transfer point. • Brief all workers on exclusion zone and spotter communication protocol before raising platform. • Record platform alignment check and stability test result on site checklist before transfer work starts. • Confirm building occupancy status — unoccupied — before platform raising. • Establish two-way radio communication between operator and spotter before ascent. | Medium (6) | • SUP — Supervise Raise scissor lift and align • WKR — Perform Raise scissor lift and align per SWMS | ⚠️ HOLD POINT 1.4 1. Do not initiate transfer until: EWP base controls are tagged out, ground spotter is in position, and gap/step dimensions are confirmed within limits (300mm horizontal / 100mm vertical) 2. Do not proceed to roof transfer until: platform is level, stable, and aligned squarely with gate opening, confirmed by spotter and written on checklist. 3. Do not raise platform to full roof height until: scissor lift base is confirmed level, stable, and on firm ground. 🛑 STOP-WORK TRIGGER • Scissor lift shows any hydraulic fluid leak, loss of pressure, or uneven raise rate during ascent. • Platform rocks, sways, or shows instability at any height during raising or at roof transfer point. • Platform misaligns with gate opening or cannot be positioned squarely without force or contact with roof edge. • EWP base controls are not tagged out or spotter is not in position before transfer • Gap between EWP platform and roof exceeds 300mm horizontal or 100mm vertical step • Wind speed exceeds EWP manufacturer OEM limit — do not transfer in high wind RISK CODE:WAH-H6 |
| 1.5 | Conduct controlled worker transfer into protected roof zone | • Fall from height — worker slip or trip during platform-to-roof transition • Struck by guardrailed gate — gate malfunction or premature closure • Fall from height — worker loss of balance on roof edge during entry briefing area • Entrapment — worker caught between platform and roof edge or gate | High (9) | Engineering Controls: • BASE CONTROLS TAGGED OUT before any worker initiates transfer step — non-negotiable. Ground spotter confirms tag-out complete and calls 'TRANSFER CLEAR' before worker moves • Confirm horizontal gap between EWP platform and roof landing does not exceed 300mm and vertical step does not exceed 100mm — re-measure if EWP has repositioned • Worker attaches lanyard to certified structural roof anchor before disconnecting from EWP anchor — worker must not be unattached at any point during transfer • Ground spotter maintains visual contact with transferring worker throughout and confirms safe landing before clearing next worker • Before gate opening, supervisor checks gate hinges, latch mechanism, and full closure seal — confirm no damage or corrosion. • Establish exclusion zone on platform and roof perimeter — minimum 2m from open gate during transfer — barricade with tape and warning signs. • Open gate slowly by hand — supervisor maintains hand contact throughout opening — stop immediately if resistance or binding detected. • Worker approaches gate in upright posture — feet shoulder-width apart — no running, jumping, or sideways movement. • Transfer one worker at a time — second worker waits on platform behind exclusion line — no concurrent movement. • Worker crosses threshold with both hands free — step deliberately onto roof edge guardrail base — pause and confirm balance before moving. • Supervisor visually confirms worker clear of gate threshold — only then signal next worker to approach gate line. • After each transfer, supervisor closes gate by hand — maintain contact throughout closure — confirm full latch engagement and lock engagement. Administrative Controls: • Verify each worker holds current White Card (CPCCWHS1001) and current Working at Heights competency (RIIOHS204E or equivalent) before transfer begins. • Obtain signed engineer confirmation that roof structure is adequate at platform-to-roof transfer point — attach to site file. • Brief supervisor on gate closure procedure and guardrail inspection routine before first transfer. • Record transfer completion, guardrail condition, and worker briefing on site safety log. | Medium (6) | • SUP — Supervise Conduct controlled worker transfer into • WKR — Perform Conduct controlled worker transfer into per SWMS | ⚠️ HOLD POINT 1.5 1. Do not initiate transfer until: EWP base controls are tagged out, ground spotter is in position, and gap/step dimensions are confirmed within limits (300mm horizontal / 100mm vertical) 2. Do not open gate until: supervisor confirms gate mechanism operates freely, latch is intact, and exclusion zone is established and clear of unauthorised personnel. 3. Do not brief workers or begin work on roof until: all workers have been transferred safely, gate is fully closed and locked, and roof edge guardrails have been walked and confirmed intact. 🛑 STOP-WORK TRIGGER • Gate binds, sticks, or does not close fully — do not force gate. • Worker stumbles, loses balance, or shows hesitation during transfer — retreat to platform and reset. • Guardrail damage, loose uprights, or missing components are found during perimeter walk — confirm repair before any roof work starts. • EWP base controls are not tagged out or spotter is not in position before transfer • Gap between EWP platform and roof exceeds 300mm horizontal or 100mm vertical step • Wind speed exceeds EWP manufacturer OEM limit — do not transfer in high wind RISK CODE:WAH-H6 |
| 1.6 | Clean condenser coils on four rooftop AHUs | • Fall from height — worker proximity to unprotected roof edge during coil cleaning • Chemical burn — contact with approved coil cleaner product • Eye injury — cleaner spray or rinse water splash during cleaning • Electrical shock — water contact with energised electrical components if unit powered | High (6) | Engineering Controls: • Confirm roof edge protection (barriers, guardrails, or netting) installed and compliant before accessing roof. • Establish exclusion zone minimum 2 m from unprotected edges; prohibit worker movement beyond zone boundary. • Wear fall arrest harness with lanyard secured to fixed anchor point rated ≥15 kN before starting cleaning. • Obtain SDS for coil cleaner on point of use; verify dilution ratio and contact time before application. • Apply coil cleaner only to fins and tubes; prevent spray contact with adjacent electrical components or energised surfaces. • Wear chemical-resistant gloves (nitrile minimum) and eye protection • Use low-pressure water rinse (≤20 bar) or compressed air; direct spray away from worker face and body. • Power down AHU unit at isolation switch before starting cleaning; verify de-energised with voltage tester. Administrative Controls: • Verify worker holds current White Card (CPCCWHS1001) and working at heights competency (RIIOHS204E or equivalent) before roof access. • Brief all workers on roof edge location, exclusion zone boundary, and harness anchor points before starting. • Record SDS location and product dilution ratio on site induction or work card. • Document fall arrest system pre-use inspection and isolation confirmation before work commences. | Medium (3) | • SUP — Supervise Clean condenser coils on four • WKR — Perform Clean condenser coils on four per SWMS | ⚠️ HOLD POINT 1.6 1. Do not access roof until: edge protection verified complete and fall arrest anchor point tested and signed off by site supervisor. 🛑 STOP-WORK TRIGGER • Worker proximity to roof edge exceeds 2 m or edge protection is breached or damaged. • Cleaner contacts energised electrical surfaces, water ingress into electrical enclosures, or voltage tester shows energised components. • Wind speed exceeds 40 km/h or worker harness/lanyard damage is observed. RISK CODE:WAH-H6 |
| 1.7 | check belts and bearings on four rooftop AHUs | • Fall from height — worker proximity to unprotected roof edge during inspection • Crush or laceration injury — rotating belt or pulley contact during contact check • Noise-induced hearing damage — prolonged exposure to bearing noise during assessment • Struck by moving component — unexpected unit restart during inspection | High (6) | Engineering Controls: • Confirm roof edge protection installed and secured before worker transfers to roof — fall arrest harness fitted and tethered to fixed roof anchor point during entire inspection. • Isolate each AHU electrically before opening belt guards or accessing rotating components — use lockout tag on disconnect switch; verify isolation with multimeter before contact. • Check belt tension using belt tension gauge; do not hand-test while unit is de-energised — record tension reading and visual wear (cracks, fraying, glazing) for each belt. • Listen for bearing noise and vibration using stethoscope or vibration meter at bearing housing only — do not place hand or fingers on rotating or moving parts. • Wear organic vapour respirator and hearing protection (foam plugs, minimum NRR 20 dB) during noise assessment — remove respiratory protection immediately after bearing assessment. • Brief worker on no-start rule: do not restart any AHU during inspection — post 'DO NOT START' sign on disconnect during work. • Record condition, wear, misalignment, and units requiring follow-up service in inspection log — photograph belt surface condition and bearing location if defect identified. • Confirm all belt guards reinstalled and secured; all AHU isolation tags removed; roof area cleared of tools and equipment before worker transfers off roof. Administrative Controls: • Verify worker holds current Working at Heights competency (RIIOHS204E or equivalent) before task starts. • Confirm roof structure and anchor points assessed and approved by site engineer before EWP access or rope access method is used. • Brief worker on location of four AHUs, roof hazards (edge location, fragile areas, obstructions), and roof emergency procedures before work starts. • make sure fall rescue plan (response time, rescue equipment, trained responder) is documented and available on site before elevated work commences. | Medium (3) | • SUP — Verify access equipment safe, approve defect list, sign off completion • WKR — Inspect per specification, photograph defects, report issues | ⚠️ HOLD POINT 1.7 1. Do not access roof or begin inspection until: roof edge protection (guardrail or certified fall arrest system) is installed and inspected by site supervisor, and fall rescue plan is confirmed available on site. 2. Do not open belt guards or access rotating components until: AHU is electrically isolated, lockout tag is applied, and isolation is verified by multimeter or equivalent test. 🛑 STOP-WORK TRIGGER • Roof edge protection is damaged, incomplete, or absent during inspection. • AHU cannot be safely isolated or isolation cannot be verified before opening guards. • Bearing noise assessment indicates vibration or noise consistent with imminent bearing failure — isolate unit and escalate to maintenance supervisor. |
| 1.8 | Document maintenance completion and prepare for return transfer | • Fall from height — worker inattention during equipment collection near roof edge • Struck by falling object — unsecured tools or materials falling during collection • Loss of situational awareness — fatigue or distraction during close-out documentation | Medium (6) | Engineering Controls: • BASE CONTROLS TAGGED OUT before any worker initiates transfer step — non-negotiable. Ground spotter confirms tag-out complete and calls 'TRANSFER CLEAR' before worker moves • Confirm horizontal gap between EWP platform and roof landing does not exceed 300mm and vertical step does not exceed 100mm — re-measure if EWP has repositioned • Worker attaches lanyard to certified structural roof anchor before disconnecting from EWP anchor — worker must not be unattached at any point during transfer • Ground spotter maintains visual contact with transferring worker throughout and confirms safe landing before clearing next worker • Before collecting tools or materials, move to centre of roof platform — minimum 2 m from all edges. • Place all tools and materials in a closed container or bag — do not carry loose items near edge. • Brief second worker (if present) on collection sequence — confirm handover of each item before movement. • Complete job sheet at workstation away from edge — do not document while standing near perimeter. • Conduct tool count against pre-work inventory — mark discrepancies on job sheet before leaving platform. • Attach lanyard to harness and anchor point — keep harness on until fully inside EWP bucket or return to ground. • Signal ground supervisor on radio — confirm ready for platform descent before moving to transfer point. • Do not collect or move tools after radio signal sent — load is frozen until ground supervisor confirms descent. Administrative Controls: • Maintain job sheet on clipboard — do not set down near edge or allow to blow away. • Record filter make, serial, and replacement date on job sheet. • Record coil condition (clean / debris / damage) and cleaning method on job sheet. • Record bearing inspection result (smooth / noise / resistance) on job sheet. • Sign and date job sheet — hand to ground supervisor at handover. • tell ground supervisor by radio: 'Work complete, tools collected, returning to ground level.' | Low (3) | • SUP — Supervise Document maintenance completion and prepare • WKR — Perform Document maintenance completion and prepare per SWMS | ⚠️ HOLD POINT 1.8 1. Do not initiate transfer until: EWP base controls are tagged out, ground spotter is in position, and gap/step dimensions are confirmed within limits (300mm horizontal / 100mm vertical) 2. Do not transfer from roof to EWP bucket until: job sheet completed, all tools accounted for, lanyard attached to anchor, and ground supervisor confirms bucket is stable and ready to receive. 🛑 STOP-WORK TRIGGER • Any tool or material is found loose near roof edge during collection. • Job sheet cannot be completed due to missing filter, coil, or bearing inspection data — do not leave platform until findings are recorded. • Worker feels fatigued or unbalanced — return to EWP bucket immediately and tell ground supervisor. • EWP base controls are not tagged out or spotter is not in position before transfer • Gap between EWP platform and roof exceeds 300mm horizontal or 100mm vertical step • Wind speed exceeds EWP manufacturer OEM limit — do not transfer in high wind RISK CODE:WAH-H6 |
| 1.9 | Conduct controlled worker return transfer from roof to scissor lift | • Fall from height — worker slip or trip during roof-to-platform transition • Struck by guardrailed gate — gate malfunction or premature closure • Fall from height — worker loss of balance exiting roof during descent • Entrapment — worker caught between roof edge and descending platform | High (9) | Engineering Controls: • BASE CONTROLS TAGGED OUT before any worker initiates transfer step — non-negotiable. Ground spotter confirms tag-out complete and calls 'TRANSFER CLEAR' before worker moves • Confirm horizontal gap between EWP platform and roof landing does not exceed 300mm and vertical step does not exceed 100mm — re-measure if EWP has repositioned • Worker attaches lanyard to certified structural roof anchor before disconnecting from EWP anchor — worker must not be unattached at any point during transfer • Ground spotter maintains visual contact with transferring worker throughout and confirms safe landing before clearing next worker • Verify scissor lift platform is level, stable, and positioned flush against roof edge before gate opening. • Confirm guardrailed gate operates freely and closes fully — test open-close cycle once before transfer commences. • Brief each worker on transfer sequence: walk straight, step down onto platform, remain on platform until gate fully closes. • Open guardrailed gate slowly and hold in open position throughout worker transfer — do not release until worker is on platform. • Transfer one worker at a time — no multiple workers on transition at once. • Direct worker to step down deliberately onto platform — no jumping or rushing. • Confirm worker is fully on platform and in safe position before closing gate. • Close guardrailed gate fully and verify closure is secure before lift descends. Administrative Controls: • Appoint one competent supervisor to direct transfer — supervisor remains visible throughout sequence. • Record worker count before transfer and confirm count on platform after final gate closure. • If transfer is delayed or interrupted, re-brief all workers on sequence before restarting. • Document any equipment failure or near-miss during transfer and report to site manager. | Medium (6) | • SUP — Supervise Conduct controlled worker return transfer • WKR — Perform Conduct controlled worker return transfer per SWMS | ⚠️ HOLD POINT 1.9 1. Do not initiate transfer until: EWP base controls are tagged out, ground spotter is in position, and gap/step dimensions are confirmed within limits (300mm horizontal / 100mm vertical) 2. Do not start worker transfer until: scissor lift platform is level, flush against roof edge, and guardrailed gate is confirmed operational and tested open-close. 3. Do not lower scissor lift until: guardrailed gate is fully closed, locked, all workers and equipment are accounted for on platform, and roof area is visually confirmed clear. 🛑 STOP-WORK TRIGGER • Scissor lift platform is not level or has shifted from roof edge contact. • Guardrailed gate is damaged, stuck, or does not close fully. • Worker loses balance, falls, or requires emergency assistance during transfer. • EWP base controls are not tagged out or spotter is not in position before transfer • Gap between EWP platform and roof exceeds 300mm horizontal or 100mm vertical step • Wind speed exceeds EWP manufacturer OEM limit — do not transfer in high wind RISK CODE:WAH-H6 |
| 1.10 | Lower scissor lift and secure at ground level | • Crush injury — scissor lift hydraulic failure or uncontrolled descent • Struck by moving platform — worker contact during lowering • Loss of stability — platform contact with obstacle or uneven floor • Entrapment — worker hand or foot contact with scissor mechanism during descent | High (6) | Engineering Controls: • Lower platform slowly using control button — do not exceed 0.5 m/s descent rate. • Position spotter at ground level with clear line of sight to platform and floor contact point. • Stop lowering immediately if platform tilts, jerks, or contacts obstacle — do not resume until cause is identified. • Confirm platform fully contact with car park floor and weight transferred — platform must not rock or shift. • Retract and lock scissor mechanism fully — engage all lock pins and confirm visual lock engagement before releasing controls. • Remove all exclusion barriers and signage — confirm area is clear of warning tape and traffic cones. • Verify all personnel are clear of platform and scissor mechanism — no hand or foot contact permitted during retraction. Administrative Controls: • Brief all personnel on ground control procedures before lowering commences. • Confirm EWP operator competency current — Working at Heights (RIIOHS204E or equivalent). • Record completion of lowering and ground-level securing in site log. | Medium (3) | • SUP — Supervise Lower scissor lift and secure • WKR — Perform Lower scissor lift and secure per SWMS | ⚠️ HOLD POINT 1.10 1. Do not retract scissor mechanism until: platform is confirmed stable and fully seated on floor by spotter sign-off. 2. Do not remove exclusion barriers until: platform is fully retracted, locked, and all personnel cleared from work area. 🛑 STOP-WORK TRIGGER • Platform descends faster than controlled rate or jerks during lowering. • Platform rocks, tilts, or does not fully contact floor at any point. • Scissor mechanism lock pins do not engage fully or lock pins are visibly damaged or missing. RISK CODE:WAH-H6 |
| 1.11 | Remove scissor lift from site and confirm roof zone clear | • Struck by moving vehicle — scissor lift towing or transport in car park • Struck by falling object — unsecured roof equipment or materials during final check • Loss of communication — unclear hand-over procedures with building contact | Medium (4) | Engineering Controls: • Verify scissor lift is empty, stable, and fully lowered before towing. Check wheel brakes are locked and no load is suspended. • Establish exclusion zone (minimum 10 m radius) around scissor lift in car park. Barricade or cone-delineate entire tow path. No pedestrian or vehicle crossing permitted. • Assign dedicated spotter. Spotter to remain visible to tow vehicle driver always. Use radio or hand signals (agreed beforehand) — never rely on shouting. • Tow scissor lift at walking speed only. Driver to sound horn before moving. Stop immediately if spotter signals stop or communication is lost. • Conduct final roof walk with building contact present. Check all equipment, materials, tools, and debris have been removed. Photograph bare roof and document in job sheet. • Check roof access gate. Confirm it closes fully, locks engage properly, and is secured. Test lock function twice. • Confirm with building contact in writing (signed job sheet and hand-over checklist) that roof zone is clear and safe. Retain copy for site records. Administrative Controls: • Obtain written confirmation from scissor lift supplier or storage facility that equipment has been received and is in acceptable condition. • Complete and sign job sheet with building contact. Document: scissor lift removal date and time, final roof condition (clear/not clear), roof access gate status (secured/not secured), and any defects or items left on roof. • Provide copy of signed job sheet and hand-over checklist to building contact and retain copy in site file. | Low (2) | • SUP — Supervise Remove scissor lift from site • WKR — Perform Remove scissor lift from site per SWMS | ⚠️ HOLD POINT 1.11 1. Do not tow scissor lift until: exclusion zone is established, spotter is briefed and positioned, and tow path is clear of all pedestrians and vehicles 2. Do not hand over to building contact until: final roof walk is complete, all equipment removed is documented, roof access gate is tested and secured, and job sheet is signed by both parties 🛑 STOP-WORK TRIGGER • Spotter loses visual contact with tow vehicle or communication fails during transport • Any equipment, material, or debris is found on roof during final walk or roof access gate fails to lock • Building contact refuses to sign hand-over sheet or raises unresolved safety concern RISK CODE:SIL-H6 |

CCVS TABLE

| Task | Critical Control | Who Checks | How Often | What They Look For |
| --- | --- | --- | --- | --- |
| Replace HVAC filters on four rooftop AHUs | Confirm HVAC isolation switch is locked OFF and no fan rotation is present. | Supervisor — before filter removal commences | Before each AHU filter removal | Isolation lock in place; no audible or visible fan movement for 30 seconds; supervisor sign-off on work permit |
| Verify scissor lift access arrangement and roof entry conditions | Supervisor visually confirms roof edge protection continuous, undamaged, and correct height before each lift deployment. | Supervisor | Before each scissor lift deployment and before occupant transfer | Site diary entry; photographic record of edge protection condition; gate operation test performed and signed |
| Raise scissor lift and align platform at roof transfer point | Check platform alignment with gate opening at full height using laser level — spotter confirms alignment visually. | Scissor lift operator and ground spotter | Before each raising cycle and before transfer gate test | Platform alignment checklist signed by operator and spotter; photographic record of platform position relative to gate opening |
| Conduct controlled worker transfer into protected roof zone | Supervisor confirms gate closure latch engagement and roof edge guardrails intact before work commences. | Supervisor | Before each worker transfer and before briefing commencement | Visual inspection record on site safety log — gate latch status, guardrail uprights intact, no visible damage, exclusion zone cleared. |
| Clean condenser coils on four rooftop AHUs | Verify fall arrest lanyard secured to tested anchor point before and during cleaning. | Site supervisor and worker (buddy check) | Before roof access and continuously during work | Visual confirmation of harness attachment; signed anchor point inspection record; worker verbal acknowledgement |
| Document maintenance completion and prepare for return transfer | Worker remains minimum 2 m from all roof edges during collection and documentation. | Second worker (if assigned) and ground supervisor via radio observation. | Continuous during tool collection and documentation phase. | Visual confirmation from ground supervisor; job sheet signed with timestamp; tool count reconciled against pre-work inventory on job sheet. |
| Conduct controlled worker return transfer from roof to scissor lift | Scissor lift platform remains level and flush against roof edge throughout transfer. | Supervisor | continuous during transfer sequence | Visual confirmation before each worker transfer; supervisor signs off platform position and gate status on transfer log. |
| Lower scissor lift and secure at ground level | Spotter confirms platform stable contact and full floor weight transfer before retraction. | EWP operator + ground spotter | Continuous during lowering and retraction; final confirmation before barrier removal. | Spotter visual sign-off; platform stable without rocking or tilting; all lock pins visibly engaged; work log entry. |
| Remove scissor lift from site and confirm roof zone clear | Dust extraction running and P2 respirator fitted before each grinding cycle | Site Supervisor | Continuous during tow operation | Spotter positioned and radio check performed before tow starts; photographic record of empty roof and secured gate; signed job sheet with building contact |

RISK RATING MATRIX — Consequence × Likelihood

| Risk Level | Description of Consequence or Impact | Consequence | Unlikely (1) | Possible (2) | Almost Certain (3) |
| --- | --- | --- | --- | --- | --- |
| HIGH | Actual/Potential fatality, disability or irreversible damage. Major structural failure. Off-site environmental discharge not contained, significant long-term environmental harm. | Major (3) | Medium (3) | High (6) | High (9) |
| MEDIUM | Actual/Potential temporary disability, MTI or LTI. Structural failure/damage, >1-day outage. On-site environmental discharge contained, minor remediation, short-term harm. | Moderate (2) | Low (2) | Medium (4) | High (6) |
| LOW | Incident requiring first aid. Environmental discharge immediately contained, minor clean-up, no short-term environmental harm. | Minor (1) | Low (1) | Low (2) | Medium (3) |
| Level | Likelihood / Probability |  |  |  |  |
| Almost Certain (3) | Occurs frequently; >66% chance of occurring |  |  |  |  |
| Possible (2) | Could happen occasionally; >33% but <66% chance of occurring |  |  |  |  |
| Unlikely (1) | May occur only in exceptional circumstances; <33% chance of occurring |  |  |  |  |
| Class / Ranking | Description / Requirements |  |  |  |  |
| High 6, 9 | STOP immediately. Implement controls. Controls recorded on a SWMS. |  |  |  |  |
| Medium 3, 4 | Planned control. Controls recorded on a SWMS. |  |  |  |  |
| Low 1, 2 | Managed via routine procedure. |  |  |  |  |
| Note — WHS Act s.18 “reasonably practicable”: Requires consideration of the likelihood of the risk, degree of harm, knowledge of the hazard, availability and suitability of controls, and cost vs. risk. If you cannot show how that decision was made, the action becomes harder to defend after an incident. |  |  |  |  |  |

| WORKER SWMS INDUCTION SIGN-OFF |  |  |  |
| --- | --- | --- | --- |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |

| WORKER SWMS AMENDMENT SIGN-OFF |  |  |  |
| --- | --- | --- | --- |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |

If work methods, site conditions, or hazards change — stop, complete this table, re-brief all workers, obtain new sign-offs before resuming. Required under WHS Regulation r.300(3).

| SWMS AMENDMENTS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Under WHS Act s18, “reasonably practicable” requires consideration of likelihood of risk, degree of harm, what the person knows about the hazard, availability and suitability of controls, cost vs risk. If you cannot show how that decision was made, the action becomes harder to defend after an incident. |  |  |  |  |  |  |
| Date | Work Activity / Task | Hazard / Risk | Risk Rating (Pre) | Controls — Hierarchy of Control | Risk Rating (Post) | Responsible |

This SWMS is a living document. It must be reviewed if work methods change, new hazards are identified or following any incident. Supervisor/Project Manager is responsible for ensuring this document remains current. NSW WHS Regulation r.300(3).

| WORKER SWMS AMENDMENT SIGN-OFF |  |  |  |
| --- | --- | --- | --- |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |
| Date: | Name: | Signature: | I confirm I have read and understood this SWMS. I will verify that critical controls are in place prior to commencing high-risk tasks and will suspend work if controls are not established. |

If work methods, site conditions, or hazards change — stop, complete this table, re-brief all workers, obtain new sign-offs before resuming. Required under WHS Regulation r.300(3).

| SWMS AMENDMENTS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Under WHS Act s18, “reasonably practicable” requires consideration of likelihood of risk, degree of harm, what the person knows about the hazard, availability and suitability of controls, cost vs risk. If you cannot show how that decision was made, the action becomes harder to defend after an incident. |  |  |  |  |  |  |
| Date | Work Activity / Task | Hazard / Risk | Risk Rating (Pre) | Controls — Hierarchy of Control | Risk Rating (Post) | Responsible |

This SWMS is a living document. It must be reviewed if work methods change, new hazards are identified or following any incident. Supervisor/Project Manager is responsible for ensuring this document remains current. NSW WHS Regulation r.300(3).